MARMARA UNIVERSITY

FACULTY OF ENGINEERING

ELECTRICAL AND ELECTRONICS ENGINEERING

ELECTRONICS 1

Simulation Project: Cockcroft-Walton Generator

Berat Asrın CAFEROĞLU – 150718056

# 1- Introduction to Cockcroft-Walton Generator

#### What Is the Cockcroft-Walton Generator?

- Cockcroft-Walton Generator is a simple voltage multiplier circuit that is designed by John Cockcroft and Ernest Walton. This generator bring them a Nobel Prize. Main idea behind this circuit is that circuit multiplies the input several times and give it to the output.

- This generator has a very special property, the input and the output are not in the same waveform. Because of the circuit characteristics the input in the form of  Alternative Current (AC) occurs in the output as Direct Current (DC).

#### Which Areas Do We Use This Generator?

- Cockcroft-Walton Generator is invented by scientists to be used as accelerator for atomic particles, and its smaller versions provided high voltages in the cathode ray tube televisions in the past. Of course the generators that creates high-voltage output needs a greater companents instead of the ones that we use in everyday electronics.

- In today’s world this type of generator is used in the devices which need high voltages to operate such as X-ray, microwave ovens etc.

- Before getting to the technical or schemetic part of this circuit let us see the picture of this generator.

Figure 1: The Cockcroft-Walton Generator

# 2- Circuit description

#### Building Block of the Cockcroft-Walton Generator: Voltage Doubler

- In fact, Cockcroft-Walton generator is based on the voltage doubler circuit. If we get many of them together in as special way we can obtain Cockcroft-Walton generator. So that, let us get familiar with the voltage doubler circuit.

- Voltage doubler is the voltage multiplier which scales the Vpeak of the AC input by 2 and gives the output in the DC. Basicly we can say that Vout = 2Vpeak. The circuit scheme of this multiplier is given below.

Figure 2: Circuit Scheme For Voltage Doubler

- We have AC input, so that to give a direction to the current the diodes are sufficient to use. Let us simulate the circuit step by step. In order to make the explaination simple the diodes are assumed as ideal, means that there is no loss on the diodes. And initially all the capacitors are uncharged.

- Assume that Vin is at the negative peak.

- In such case the D1 diode is forward biased because the P side has greater voltage than N side, that is, this diode is conducting.

- This case allows to charge the C1 capacitor up to Vin.

- Meanwhile the D2 diode is also forward biased just for one time in this half wave, and the reason will be explained in following steps.

- The capacitor C2 is charged up to Vin.

- Then, assume that Vin is at the positive peak.

- In such case the D1 diode is reversed biased because the P side has lower voltage than N side, that is, this diode is not conducting.

- Remember that the C1 was charged up to Vin now it supports the source and the node at the plus side of C1 has Vin + Vin = 2Vin.

- So that, the D2 is forward biased, and it conducts the current. The node at the N side of diode has the voltage of 2Vin because there is no loss on the diode due to diodes are ideal.

- In the step 3 we said that C1 is charged up to Vin just for one time because now it will be charged up to 2Vin, and now the input voltage is doubled at the output as you can see. Notice that while charging the C2 the C1 will be discharging.

- Let us repeat the same procedure but this time we continue from the step 10 where the capacitors are charged. C1 < Vin because it charged the other capacitor, C2 = 2Vin. Again assume that the diodes are ideal.

- We have Vin that is at the negative peak again.

- The P side of D1 diode has Vin again, and remember that C1 < Vin, that is, the N side has a voltage less than Vin. Thus, the diode D1 is forward biased.

- C1 is being charged now up to Vin.

- Unlike step 4, the D2 diode reversed biased now. So that, the current coming from the plus side of the voltage source cannot return to the minus side of the voltage source.

- In such condition if we connect a load resistor to the Vout. The C2 will be source to this load. And at the and of the negative half-wave the voltage stored in the C2 will be less than 2Vin. So that, C2 < 2Vin.

- Now let us assume that the Vin is at the positive peak again.

- Now capacitor C1 supports the voltage source and the plus side of the capacitor will be 2Vin.

- In this case, the D1 is reversed biased, and it does not conduct the electricity because the N side has greater voltage value than P side.

- And the D2 diode is forward biased, because the P side has greater voltage.

- At the end of the negative half-wave we said that C2 < 2Vin, and the D2 conducts the electricity. So that, the capacitor C2 is charged up to 2Vin again.

- Let us simulate the voltage doubler and see the input output relationship.

Figure 3: Voltage Doubler Circuit

Figure 4: Output Voltage According To Time

The source V1 has peak value of 10 volts, frequency of 60 Hz, and the phase of 0 radians. Theoretically we expect the output to be Vout = 2V1 = 20 volts. The output of the simulation is given below.

- Here we see that Vout = 19.310003 volts. Actually the results can be considered as great because we do not have any ideal diodes in the real life, and note that the 1N4148 has the forward voltage around 0.65 volts. The improvement ideas to decrease the error percentage will be given in the following parts after the Cockcroft-Walton generator.

  - Percentage Error=20-19.31000320×100%=3.449985%

  - This error mostly occurs because of the drop on the D2 diode.

- This type of voltage multipliers are called as half-wave voltage multiplier because only one half of the input source charges the C2. Now we will discuss how to design a full-wave voltage multiplier shortly.

Figure 5: Full-Wave Voltage Doubler Circuit

In order to have full-wave voltage multiplier just connect two voltage doubler to the same ground. The important point here is that because of we connect the plus side of V2 to the ground there is π radians phase angle between them. In other words, the full-wave voltage multiplier can be created by using 2 half-wave voltage multipliers with different polarities. Notice that capacitor C2 is being charged in the both half-waves.

Figure 6: Output of the Full-Wave Voltage Doubler

- The output of the full-wave voltage doubler is given above at the same instant with half-wave voltage doubler.

  - Percentage Error=20-19.35799420×100%=3.21003%

  - The error is decreased because we design full-wave voltage multipliers to have less ripples in the output. So, because of the ripples have been decreased the average output have been increased, that is, the DC value have been increased too.

  - If the peak value of the input source were more greater value the percentage error in the half-wave voltage doubler would be also greater.

- The main advantage of the voltage multiplier circuits is that these circuits allow to have high voltages using the low input voltages without using expensive transformer or OP-AMP. In other words, they are cost efficient.

- The main disadvantage of the voltage multiplier circuits is that to get a stable DC output the capacitance values of the capacitor should be arranged properly. Otherwise, the ripples may be occur in the output, and the form becomes other than DC. To prevent these ripples the full-wave voltage multiplier is recommended to use.

#### Cockcroft-Walton Generator Circuit

- In the previous part we studied the voltage doubler, and the Cockcroft-Walton generator is just the circuit consists of cascaded voltage doublers.

Figure 7: A Four-Stage Half-Wave Cockcroft-Walton Generator Circuit

The reference model that I searched uses a half-wave voltage multiplier type of the generator. In this part, we will analyze the reference circuit, and then my improvements on the circuit will be given in the coming parts.

- Here every one voltage doubler is called stage and number of stages is represented by N. Remember that the voltage doubler consists of two capacitors and two diodes. Therefore, the number of stages can be calculated easily by the formula:

  - N=Number of Diodes2

  - For example, in figure 7 we have 8 diodes, let us divide it 2. Then, we have four stages.

- Assuming the diodes in the design are ideal and there is no voltage loss on them. In that case, we can say that:

  - Vout=2NVpeak=NVpeak-to-peak

  - The Vpeak represent that the peak point of the source called V1.

  - The Vpeak-to-peak represent that the peak-to-peak voltage difference of the source called V1.

- Of course there will be less output voltage than expected in real life because,

  - We do not have any ideal diodes in real life. So that, there is always a voltage loss on them.

  - The capacitance values of the capacitors affect the amount of ripples. So, this value should be arranged wisely according to the needed output.

  - No need to mention but as we make the system more closer to the expected output the cost of the circuit will be more expensive due to the component selections.

- Now let us simulate the reference design without connecting any load to the output and any improvments on the reference circuit. The components are as follows:

  - 1000pF Capacitors

  - AC Signal Source (5 V Peak-To-Peak, 1kHz Frequency, 0 Radian Phase)

  - 1N4148 Diodes

Figure 8: Output of Reference Design Without Load

Figure 9: Zoomed Output of Reference Design Without Load

Figure 10: Ripple In the Reference Design

As you can see from the figure 8 the output of the generator is Vout = 17.795338 volts. If we check percentage error and efficiency of the circuit without load:

  - Number of Stages N=4, Vpeak-to-peak=5 volts

  - Expected Output=NVpeak-to-peak=4×5=20 volts

  - Percentage Error=20-17.79533820×100%=11.02331%

  - Efficiency=17.79533820×100%=88.97669%

- Even though we said the output is DC with these capacitor values and diodes there is a lot ripple. Which can be found using peak-to-peak difference at the output. It is seen that LTSpice calculates the difference between cursors and the result is:

  - Amount of Ripple=Vr=17.795338-17.589624=205.71327 mV

- Without load at the output everything is fine because the only deal of the circuit is to charge capacitors but if we connect a load, the current will also pass through the load resistor. So, let us check what happens in the output if we connect an 100 kΩ load.

Figure 11: Reference Design With Load Resistor

Figure 12: Output of Reference Design With Load

As you can see from the figure 12 the output with load resistor is far away from what we expected and which is a big problem. Even if we connect a big resistor like 100 kΩ the output is not satisfying. So that, the reference design and the values of components must be rearranged.

# 3- Circuit Improvements

- In this part we will discuss how the circuit can be more stable and how to output converge to the DC with existence of load resistor. At the end of the this part the comparison between reference circuit and improved circuit will be discussed. Shortly, this part contains my improvement ideas about reference design.

#### Step 1: Use Full-Wave Voltage Multiplier Instead Of Half-Wave

- As it is mentioned before there is several ways to eliminate the ripples and get DC output. The first of them is connecting the voltage multiplier stages in full-wave order.

Figure 13: Two-Stage Full-Wave Cockcroft–Walton Generator

To design full-wave voltage multiplier two voltage multiplier circuits must be connected with opposite polarities.

Figure 14: Four-Stage Full-Wave Cockcroft–Walton Generator

In the figure 13 the transformer is used to explain the design of the full-wave voltage multiplier. In my improvement the transformer will not be exist to decrease the cost of the circuit. Instead of transformer two AC voltage sources with opposite polarities, in other words sharing the same ground, will be used.

Figure 15: Output of the Four-Stage Full-Wave Cockcroft-Walton Generator

Let us see the output without connecting the load.

Figure 16: Zoomed Output of the Four-Stage Full-Wave Cockcroft-Walton Generator

Figure 17: Ripple In the Improved Design

Let us check the output efficiency and percentage error.

  - Number of Stages N=4, Vpeak-to-peak=5 volts

  - Expected Output=NVpeak-to-peak=4×5=20 volts

  - Percentage Error=20-18.18527220×100%=9.07364%

  - Efficiency=18.18527220×100%=90.92636%

- Let us calculate the amount of ripple at the output without load resistor by looking the difference between peak points from LTSpice.

  - Amount of Ripple=Vr=18.185272-18.174767=10.505676 mV

Figure 18: A Four-Stage Full-Wave Cockcroft–Walton Generator With Load

Now the 100-kΩ load resistor is connected to the output of the generator. The circuit diagram is given below.

Figure 19: Output of Step 1 With Load

- The figure 19 shows the output with load. Changing half-wave to full-wave makes the ripples at the output less without load but as you can see from the figure it is not enough to stabilize the output when the load resistor exists. In that case the uncharging time of capacitors should be extended. The next step of the improvements includes this because we would like to have an stable output with load too.

#### Step 2: Adjust Values of Capacitors

- We know that the charging and uncharging times depends on the time constant (τ).

  - τ=R×C

  - Here R is equivalent resistor, and the C is equivalent capacitance of circuit.

- In that case the time constant should have greater value because as the time constant becomes bigger, also the uncharging time of the capacitors extends.

- To make longer this time there is two choice:

  - We can increase the value of load resistor. But it will also make the output current less, and of course in the other day we may want to connect another device with another load resistor value. So that, changing the load resistor is not a good idea for now.

  - The second way is to increase the capacitance value of the capacitors.

- Now as a addition to the step 1, we will make the capacitance values greater than before.

  - Remember that we want the output to be in DC form and as close as possible to 20 volts.

    - No need to mention but peak value of the source was 2.5 volts or peak-to-peak voltage was 5 volts and the number of stages was 4. From here we say the thoretically the output should be 20 volts using the given formula Vout=NVpeak-to-peak=5×4=20 volts

- Most important part here is that if we do not want to connect a load, smaller capacitance values would be better because already system’s energy consumption is very low to uncharge the capacitors fully. But in that case if we connect load the output is not satifying. On the other hand, if we are going to connect a load we need to adjust the values of capacitor, but in that case the charging and uncharging times will be extended. The extended charging time will occur a problem which is lower output without load (In the coming parts this problem will be solved by improvements). But the advantage of the increasing capacitances is that the output value will be in DC, and stable with load resistor.

- Capacitance values are adjusted to 100 μF decrease the ripple at the output when the load is connected.

Figure 20: Capacitance Values Changed

Figure 21: Output Without Load

Figure 22: Zoomed Output Without Load

As you can see from the figures numbered 21 and 22 because of the capacitance values of capacitors has been increased the charging time is extended and at the time interval we chose the capacitors are still charging.

- Already this improvement has been made to obtain DC output in case of load resistor existance. Before getting that point let us see what the output will be if we wait enough.

Figure 23: More Waited For Output

As the figure 23 shows us more we wait, more voltage at the output occurs (Pay attention to the time scale). In other words, as the time converges to the infinity, the voltage at the output converges to the 20 volts (But do not forget the loss on the diodes).

Figure 24: Output With Load Resistor

Now the load resistor with 100 kΩ will be connected to the output.

- The figure 24 shows us the output voltage and the current passing through the 100-kΩ resistor. Please notice that it clearly DC output and there is no noticeable ripple, thanks to the higher capacitance values. If we wait long enough there will be a loss on the diodes but the output will converge to the 20 volts.

- The only penalty of this improvement is obvious which is we need to wait so long to get higher output voltages. In the next step of improvements this problem will be solved.

#### Step 3: Change Diodes With Schottky Diode

- One reason the problem occurred in step 2 is the high-frequency AC signal source we use. Remember that the source has the frequency of 1 kHz, and the diode used in the design with the name 1N4148 needs around 0.65 volts to be forward biased. This is a huge value for high frequencies. Therefore, the model of the diodes needs to be changed with another type of diode with faster respond level. At that moment diode type called Schottky diode is the best choice.

- Schottky diodes are the fastest diodes in switching process and respond levels. Usually 0.15 volts – 0.45 volts is needed to be forward biased. So that, usage of Schottky diode in the circuit will increase the output level because the capacitors will be start to charge themselves at the lower voltage levels instead of 0.65 volts.

Figure 25: Circuit Scheme After Step 2

Let us change all the diodes with the BAT54 Schottky diode. The datasheets of the components will be given at the end of the document.

Figure 26: Output Without Load

Figure 27: Zoomed Output Without Load

Notice that, the capacitors are still charging at the moment we test in the figure 27 but let us compare the effect of improvement with before.

  - The figure 22 shows the result at the output before this improvement. In that figure, at the time 4.95 s we have output of 16.473845 volts.

  - Now take a look to the figure 27 which shows the output after the improvement. In the figure, at the time 4.95 s we have output of 18.802742 volts.

- So, it is seen that the diode selection has a huge role on the result. The diodes with fast-respond levels are better in this design.

Figure 28: Output With Load

Now let us connect a load of 100 kΩ resistor to the output.

- The figure numbered 28 shows the output of the generator with 100-kΩ load resistor. The best side of this improvement is that even if we connect a resistor to the output the voltage at the node called out remains almost the same. But before this improvement we saw that the voltage at out node is less in case of load existance.

  - To compare the results please look at the figures numbered 24 and 28.

# 4- Comparison Between Reference Circuit and Improved Circuit

- In this part the circuit will be tested under different conditions and the results will be interpreted. Shortly, this part includes the comparison between reference design and my improved design.

#### Circuit Scheme Design Differences

Figure 29: Reference Circuit Design

Figure 30: Improved Circuit Design

| Improvement Area | Reference Design | Improved Design |
| --- | --- | --- |
| Charging | Half-Wave Voltage Multipler | Full-Wave Voltage Multipler |
| Capacitor | 1000 pF | 100 μF |
| Diode | Normal Diode: 1N4148 | Schottky Diode: BAT54 |

Table 1: Main Changes

#### Test 1: Test the Circuit Without Any Load Resistor

Figure 31: Output of Reference Design Without Load

In this section the circuits will be remain the same, that is, there is no load resistor at the node called out. The results will be compared using percentages at the end of the LTSpice results. Note that capacitors are still charging but we will consider the two designs at the same time intervals without waiting the capacitors charge themselves fully.

Figure 32: Zoomed Output of Reference Design Without Load

Figure 33: Ripple Calculation of Reference Design Without Load

Figure 34: Output of the Improved Design Without Load

Figure 35: Zoomed Output of the Improved Design Without Load

Figure 36: Ripple Calculation of Improved Design Without Load

- Note that voltage goes upward in the figure 36 which is related to the improved design because the capacitors are still charging.

- The data in the table given below are related to the same time interval which is between t = 4.99s and t = 5s.

|  | Reference Design | Improved Design | Related Figures |
| --- | --- | --- | --- |
| Output Form | Close to DC | DC | 31 and 34 |
| Output Voltage | 17.795221 Volts (max) and 17.589518 Volts (min) | 18.804565 Volts | 33 and 35 |
| Percentage Error According to the Expected Output (20 Volts) | 11.023895% (Calculated using max value) | 5.977175% | - |
| Ripple Amount (Vr) (Calculated by LTSpice Based On Maximum and Minimum Data) | 205.70374 mV | 3.7856442 μV | 33 and 36 |

Table 2: Results of Test 1

- It is clearly seen that improvements which is made on the circuit affect the output in a positive way, and the output get closer to the voltage level we desire.

#### Test 2: Test the Output With Higher Input Voltage and Without Load

- In this test section we will consider the circuit with higher input voltage levels and without load resistor to see how the error rates changes with the input voltage. Shortly, we desire to observe the relationship between percentage error and input voltage.

- The input voltage has peak value of 100 volts or peak-to-peak value of 200 volts. The frequency is again 1 kHz, and the phase angle of 0 radians. There is no load resistor.The number of stages is N = 4.

Figure 37: Output of Reference Design

Using the formula mentioned before the output is expected to be 800 volts.

  - Vexpected=NVpeak-to-peak=4×200=800 volts

Figure 38: Zoomed Output of Reference Design

Figure 39: Output of Improved Design

- Samples are taken from the interval t = 4.99 s and t = 5 s.

- As you can see at the output of the reference design there is a ripple with the value of 2.45 volts which cannot be negligible, and this ripple causes error in the end. But in the improved design as you can see from the figure numbered 39 the output is in the DC form.

|  | Reference Design | Improved Design | Related Figures |
| --- | --- | --- | --- |
| Output Form | Close to DC But With Ripples | DC | 37 and 39 |
| Output Voltage | 783.89429 Volts (max) and 781.44299 Volts (min) | 798.40234 Volts | 38 and 39 |
| Percentage Error According to the Expected Output (800 Volts) | 2.01321375% (Calculated using max value.) | 0.1997075% | - |

Table 3: Results of Test 2

- This test shows us as we increase the input voltage, we get less percentage error in the output. With the input given in the test procedure the error of the improved design is very close to the zero.

#### Test 3: Test the Output With Low-Valued Load Resistor

- In this test the designs will be compared with the existence of high current consumption at the output due to low-valued resistor.

- The load resistor which is connected to the output node called out has the value of 10kΩ. The input has peak value of 2.5 volts and peak-to-peak value of 5 volts again with 1 kHz frequency and 0 radians phase angle.

Figure 40: Output of Reference With Load

Samples are taken from the interval t = 4.99 s and t = 5 s.

Figure 41: Output of Improved Design With Load

|  | Reference Design | Improved Design | Related Figures |
| --- | --- | --- | --- |
| Output Form | Not Even Closer to DC | DC With Very Small Ripples | 40 and 41 |
| Output Voltage | 48.236966 mVolts (max) and -19.03772 mVolts (min) | 17.219463 Volts (max) and 17.193911 Volts (min) | 40 and 41 |
| Percentage Error According to the Expected Output (20 Volts) | 99.75881517% (Calculated using max value) | 13.902685%(Calculated using max value) | - |
| Ripple Amount (Vr) (Calculated by LTSpice Based On Maximum and Minimum Data) | 67.274686 mVolts | 25.55275 mVolts | 40 and 41 |

Table 4: Results of Test 3

- As you can see from the data included in the table there is a huge difference between the reference design and improved design.

- The reference design is too sensitive to the load resistor connections but the design I improved can still work efficiently and effectively with load resistor. With 10-kΩ load resistor efficiency percentages are as follows:

  - Reference Design  0.24118483%

  - Improved Design  86.097315%

#### Test 4: Test the Output With Higher Input Voltage and With Low-Valued Resistor

- In this test procedure:

  - Voltage Sources have 200 volts peak-to-peak voltage, 1 kHz frequency, 0 radian phase angle.

  - Load resistor with the value of 10-kΩ.

  - Samples are taken from the interval t = 4.99 s and t = 5 s.

Figure 42: Output of Reference Design

Remember that Number of stages is N = 4.  Vexpected=800 volts

Figure 43: Output of Reference Design

Figure 44: Output of Improved Design

|  | Reference Design | Improved Design | Related Figures |
| --- | --- | --- | --- |
| Output Form | Not Even Closer to DC | DC With Very Small Ripples | 42 and 44 |
| Output Voltage | 6.1246419 Volts (max) and -222.6864 mVolts (min) | 777.84412 Volts (max) and 776.53143 Volts (min) | 42 and 43 |
| Percentage Error According to the Expected Output (800 Volts) | 99.23441976% (Calculated using max value) | 2.769485%(Calculated using max value) | - |
| Ripple Amount (Vr) (Calculated by LTSpice Based On Maximum and Minimum Data) | 6.3473283 Volts | 1.3126831 Volts | 42 and 43 |

Table 5: Results of Test 4

- Efficiency of the designs are as follows:

  - Reference Design  0.76558024%

  - Improved Design  97.230515%

# 5- Conclusion and Comments

#### Last Reminders and Remarks

Figure 45: Improved Design With Stages

At the beginning we explained that the Cockcroft-Walton Generator consists of cascaded voltage doublers, and remember that every single voltage doubler part was called a stage. Then, we should notice that at the end of the every stage the gain of voltage multiplier changes.  Let us take a look.

- As you can see from the figure 45 the output of the every voltage doubler is named with the related stage number.

  - If the stage is numbered with 1 which means that at that node N = 1.

Figure 46: Outputs of Stages

The overall output is located at the Stage_4. So that, the output value of the generator is calculated using N = 4.

#### Summary With My Comments About Reference Circuit and Improved Circuit

Figure 47: Reference Design

The reference Cockcroft-Walton Generator is very handy circuit design to obtain high voltages using low voltages. In my opinion, if the only goal is to increase the voltage the reference design can be used because without load resistor the ripple amounts are acceptable and the output is close to the DC. But the weakest point of this design occurs when you connect a load resistor to the output. It can work properly with high resistor values but even in that situation the result is not satisfying. Thus, the reference design needs to be redesigned. The reference design that I used is given below.

Figure 48: My Design

I investigated the weak points and rearranged the values and changed the components used in the original (reference) design, and the circuit I designed has become as given below.

- In my design the output is stable even if you connect a load resistor to the output, and according to my test the efficiency of the circuit is always higher than 85%. This generator circuit can be used to produce high voltages using low voltage inputs in the devices that requires high voltage values to work properly. Another reason to use this design is cost efficiency because by using this circuit you do not need transformer which is very expensive component. So that, the design is suitable for television sets, home appliances etc.

# References

- Reference Design of Cockcroft-Walton Generator: https://www.allaboutcircuits.com/textbook/semiconductors/chpt-3/voltage-multipliers/

- History and Short Description of Cockcroft-Walton Generator: https://www.nms.ac.uk/explore-our-collections/stories/science-and-technology/cockcroft-walton-generator/

- Voltage Multiplier Basics: https://www.electronics-tutorials.ws/blog/voltage-multiplier-circuit.html

- Voltage Multiplier and Cockcroft-Walton Generator: https://en.wikipedia.org/wiki/Voltage_multiplier

- Cockcroft-Walton Generator: https://en.wikipedia.org/wiki/Cockcroft%E2%80%93Walton_generator

- Basics of RC Circuits: https://www.electronics-tutorials.ws/rc/rc_1.html

- Datasheet of 1N4148: https://www.vishay.com/docs/81857/1n4148.pdf

- Datasheet of BAT54: https://www.vishay.com/docs/85508/bat54.pdf